EX NO: 8

POWER SPECTRAL DENSITY (PSD) ANALYSIS OF PHASE MODULATED (PM) SIGNALS USING MATLAB

Objectives

i) To generate a Phase Modulated (PM) signal in MATLAB using specified carrier and message

signal parameters.

PROGRAMMING CODE:

clc;

clear;

close all;

% Given Parameters

Am = 1;          % Message signal amplitude

Ac = 2;          % Carrier signal amplitude

fm = 100;        % Message frequency in Hz

fc = 1000;       % Carrier frequency in Hz

kp = pi/2;       % Phase sensitivity in radians/volt

% Time vector

fs = 100000;     % Sampling frequency

t = 0:1/fs:0.02; % Time duration

% Generate Message Signal

m = Am * cos(2*pi*fm*t);

% Generate Carrier Signal

c = Ac * cos(2*pi*fc*t);

% Generate Phase Modulated Signal

s = Ac * cos(2*pi*fc*t + kp*m);

% Plot Signals

figure;

subplot(3,1,1);

plot(t,m);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,1,2);

plot(t,c);

title('Carrier Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,1,3);

plot(t,s);

title('Phase Modulated (PM) Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

OUTPUTS:

OBJECTIVE:

ii) To compute the frequency spectrum of the PM signal using the Fast Fourier Transform (FFT).

PROGRAMMING CODE:

clc;

clear;

close all;

% Given Parameters

Am = 1;          % Message signal amplitude

Ac = 2;          % Carrier signal amplitude

fm = 100;        % Message frequency in Hz

fc = 1000;       % Carrier frequency in Hz

kp = pi/2;       % Phase sensitivity in radians/volt

% Sampling Parameters

fs = 10000;      % Sampling frequency

T = 0.1;         % Duration in seconds

t = 0:1/fs:T-1/fs;

% Generate Message Signal

m = Am * cos(2*pi*fm*t);

% Generate Carrier Signal

c = Ac * cos(2*pi*fc*t);

% Generate PM Signal

s = Ac * cos(2*pi*fc*t + kp*m);

% Compute FFT

N = length(s);

S = fft(s);

S_mag = abs(S)/N;

% Single-sided spectrum

P1 = S_mag(1:N/2+1);

P1(2:end-1) = 2*P1(2:end-1);

% Frequency Axis

f = fs*(0:N/2)/N;

% Plot Message Signal

figure;

subplot(3,1,1);

plot(t,m);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

% Plot PM Signal

subplot(3,1,2);

plot(t,s);

title('Phase Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

% Plot Frequency Spectrum

subplot(3,1,3);

plot(f,P1);

title('Frequency Spectrum of PM Signal Using FFT');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

xlim([0 2000]);

grid on;

OUTPUTS:

OBJECTIVE:

iii) To calculate and plot the Power Spectral Density (PSD) of the PM signal using MATLAB.

PROGRAMMING CODE:

clc;

clear;

close all;

% Given Parameters

Am = 1;          % Message signal amplitude

Ac = 2;          % Carrier signal amplitude

fm = 100;        % Message frequency in Hz

fc = 1000;       % Carrier frequency in Hz

kp = pi/2;       % Phase sensitivity in radians/volt

% Sampling Parameters

fs = 10000;      % Sampling frequency in Hz

T = 0.1;         % Duration in seconds

t = 0:1/fs:T-1/fs;

% Generate Message Signal

m = Am * cos(2*pi*fm*t);

% Generate PM Signal

s = Ac * cos(2*pi*fc*t + kp*m);

% Calculate Power Spectral Density using Welch's Method

[pxx, f] = pwelch(s, [], [], [], fs);

% Plot Message Signal

figure;

subplot(3,1,1);

plot(t, m);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

% Plot PM Signal

subplot(3,1,2);

plot(t, s);

title('Phase Modulated (PM) Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

% Plot Power Spectral Density

subplot(3,1,3);

plot(f, 10*log10(pxx + eps));

title('Power Spectral Density of PM Signal');

xlabel('Frequency (Hz)');

ylabel('PSD (dB/Hz)');

xlim([0 2000]);

grid on;

OUTPUT:

OBJECTIVE

iv) To identify the carrier component and sideband frequencies present in the PM spectrum.

PROGRAMMING CODE:

clc;

clear;

close all;

% Given Parameters

Am = 1;          % Message signal amplitude

Ac = 2;          % Carrier signal amplitude

fm = 100;        % Message frequency in Hz

fc = 1000;       % Carrier frequency in Hz

kp = pi/2;       % Phase sensitivity in radians/volt

% Sampling Parameters

fs = 10000;      % Sampling frequency

T = 0.1;         % Duration in seconds

t = 0:1/fs:T-1/fs;

% Generate Message Signal

m = Am * cos(2*pi*fm*t);

% Generate PM Signal

s = Ac * cos(2*pi*fc*t + kp*m);

% Compute FFT

N = length(s);

S = fft(s);

P2 = abs(S)/N;

% Single-sided amplitude spectrum

P1 = P2(1:N/2+1);

P1(2:end-1) = 2*P1(2:end-1);

% Frequency vector

f = fs*(0:N/2)/N;

% Find Peaks in Spectrum

[pks, locs] = findpeaks(P1, f, ...

'MinPeakHeight', 0.05, ...

'MinPeakDistance', 50);

% Display Peak Frequencies

fprintf('Carrier Frequency = %d Hz\n', fc);

fprintf('Message Frequency = %d Hz\n\n', fm);

fprintf('Identified Carrier and Sideband Frequencies:\n');

for i = 1:length(locs)

fprintf('Frequency = %.2f Hz, Amplitude = %.4f\n', ...

locs(i), pks(i));

end

% Plot Frequency Spectrum

figure;

plot(f, P1, 'b', 'LineWidth', 1.5);

hold on;

plot(locs, pks, 'ro', 'MarkerFaceColor', 'r');

title('PM Spectrum: Carrier and Sidebands');

xlabel('Frequency (Hz)');

ylabel('Amplitude');

xlim([500 1500]);

grid on;

legend('PM Spectrum', 'Detected Peaks');

OUTPUTS:

OBJECTIVE:

v) To analyze the effect of phase modulation on the spectral distribution and bandwidth of the transmitted signal.

PROGRAMMING CODE:

clc;

clear;

close all;

% Given Parameters

Am = 1;           % Message signal amplitude

Ac = 2;           % Carrier signal amplitude

fm = 100;         % Message frequency in Hz

fc = 1000;        % Carrier frequency in Hz

kp = pi/2;        % Phase sensitivity in radians/volt

% Sampling Parameters

fs = 10000;       % Sampling frequency in Hz

T = 0.1;          % Duration in seconds

t = 0:1/fs:T-1/fs;

% Generate Message Signal

m = Am * cos(2*pi*fm*t);

% Calculate Phase Deviation

beta = kp * Am;

% Generate PM Signal

s = Ac * cos(2*pi*fc*t + kp*m);

% Calculate FFT

N = length(s);

S = fft(s);

P2 = abs(S)/N;

% Single-Sided Spectrum

P1 = P2(1:N/2+1);

P1(2:end-1) = 2*P1(2:end-1);

% Frequency Axis

f = fs*(0:N/2)/N;

% Find Significant Spectral Components

threshold = max(P1) * 0.01;

idx = find(P1 >= threshold);

% Calculate Approximate Bandwidth

if ~isempty(idx)

f_low = min(f(idx));

f_high = max(f(idx));

bandwidth = f_high - f_low;

else

f_low = 0;

f_high = 0;

bandwidth = 0;

end

% Display Results

fprintf('Carrier Frequency = %d Hz\n', fc);

fprintf('Message Frequency = %d Hz\n', fm);

fprintf('Phase Deviation = %.2f radians\n', beta);

fprintf('Lower Significant Frequency = %.2f Hz\n', f_low);

fprintf('Upper Significant Frequency = %.2f Hz\n', f_high);

fprintf('Approximate Bandwidth = %.2f Hz\n', bandwidth);

% Plot Message Signal

figure;

subplot(3,1,1);

plot(t,m);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

% Plot PM Signal

subplot(3,1,2);

plot(t,s);

title('Phase Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

% Plot Frequency Spectrum

subplot(3,1,3);

plot(f,P1,'b','LineWidth',1.2);

hold on;

yline(threshold,'r--','1% Threshold');

title('Spectral Distribution of PM Signal');

xlabel('Frequency (Hz)');

ylabel('Amplitude');

xlim([500 1500]);

grid on;

OUTPUTS:

OBJECTIVE:

vi)To investigate the influence of modulation index on the Power Spectral Density characteristics of the PM signal.

PROGRAMMING CODE:

clc;

clear;

close all;

% Given Parameters

Am = 1;                    % Message signal amplitude

Ac = 2;                    % Carrier signal amplitude

fm = 100;                  % Message frequency in Hz

fc = 1000;                 % Carrier frequency in Hz

kp_values = [0.5 1 2 3];  % Phase sensitivity values in rad/V

% Sampling Parameters

fs = 10000;                 % Sampling frequency

T = 0.1;                    % Duration in seconds

t = 0:1/fs:T-1/fs;

% Generate Message Signal

m = Am*cos(2*pi*fm*t);

% Create Figure

figure;

for i = 1:length(kp_values)

% Calculate Modulation Index

beta = kp_values(i)*Am;

% Generate PM Signal

s = Ac*cos(2*pi*fc*t + kp_values(i)*m);

% Calculate PSD using Welch's Method

[pxx,f] = pwelch(s, [], [], [], fs);

% Plot PSD

subplot(2,2,i);

plot(f,10*log10(pxx + eps),'LineWidth',1.2);

title(['PM Signal PSD, Modulation Index = ',num2str(beta)]);

xlabel('Frequency (Hz)');

ylabel('PSD (dB/Hz)');

xlim([500 1500]);

grid on;

% Display Modulation Index

fprintf('Phase Sensitivity = %.2f rad/V, ',kp_values(i));

fprintf('Modulation Index = %.2f\n',beta);

end

sgtitle('Effect of Modulation Index on PM Signal PSD');

OUTPUTS: