EXP NO:2 CHARACTERISTICS STUDY OF SIDE BAND POWER ANDFREQUENCIES IN AM USING MATLAB

Objectives:

i) To generate an Amplitude Modulated (AM) signal in MATLAB using specified carrier and

modulating signal parameters.

ii) To determine and verify the upper sideband (USB) and lower sideband (LSB) frequencies of

the AM signal using frequency spectrum analysis in MATLAB.

iii) To calculate the carrier power, sideband power, and total transmitted power of the AM signal

for a given modulation index using MATLAB.

iv) To analyze the effect of varying the modulation index on sideband power distribution and

overall transmission efficiency through MATLAB simulation.

Objective 1:

To generate an Amplitude Modulated (AM) signal in MATLAB using specified carrier and

modulating signal parameters

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

Am=1;

fm=500;

Ac=5;

fc=5000;

fs=20*fc;

t=0:1/fs:4/fm;

m = Am*sin(2*pi*fm*t);

c = Ac*sin(2*pi*fc*t);

mu = Am/Ac;

am = Ac*(1+mu*sin(2*pi*fm*t)).*sin(2*pi*fc*t);

dem = abs(hilbert(am)); dem = dem-mean(dem);

N=length(am); f=linspace(-fs/2,fs/2,N);

AM_FFT=abs(fftshift(fft(am)))/N;

USB=fc+fm; LSB=fc-fm;

Pc=Ac^2/2; Psb=(mu^2*Pc)/4;

figure('Name','AM Modulation','NumberTitle','off');

subplot(3,2,1), plot(t,m), title('Message'), grid on

subplot(3,2,2), plot(t,c), title('Carrier'), grid on

subplot(3,2,3), plot(t,am), title('AM Signal'), grid on

subplot(3,2,4), plot(t,dem), title('Demodulated'), grid on

subplot(3,2,5), plot(f,AM_FFT), xlim([fc-3*fm fc+3*fm]), title('Sidebands'), grid on

subplot(3,2,6), bar([Pc Psb Psb]), set(gca,'XTickLabel',{'Carrier','USB','LSB'}),

title('Power'), grid on

fprintf('μ=%.2f | USB=%d Hz | LSB=%d Hz | Pc=%.2f W | Psb=%.3f

W\n',mu,USB,LSB,Pc,Psb);

OUTPUT (GRAPH)

Objective 2:

To determine and verify the upper sideband (USB) and lower sideband (LSB) frequencies of the

AM signal using frequency spectrum analysis in MATLAB.

Program:

clc;

clear;

close all;

% Signal parameters

Am = 1;

fm = 500;

Ac = 5;

fc = 5000;

fs = 20*fc;

% Time vector

t = 0:1/fs:4/fm;

% Generate message and carrier signals

m = Am*sin(2*pi*fm*t);

c = Ac*sin(2*pi*fc*t);

% Generate AM signal

mu = Am/Ac;

am = Ac*(1 + mu*sin(2*pi*fm*t)).*sin(2*pi*fc*t);

% FFT analysis

N = length(am);

Y = fft(am);

P = abs(fftshift(Y))/N;

f = (-floor(N/2):ceil(N/2)-1)*(fs/N);

% Calculate USB and LSB frequencies

USB = fc + fm;

LSB = fc - fm;

% Display results

fprintf('Carrier Frequency = %d Hz\n', fc);

fprintf('Message Frequency = %d Hz\n', fm);

fprintf('Upper Sideband (USB) = %d Hz\n', USB);

fprintf('Lower Sideband (LSB) = %d Hz\n', LSB);

% Plot frequency spectrum

figure;

plot(f, P, 'b', 'LineWidth', 1.2);

xlim([fc-2*fm fc+2*fm]);

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('AM Signal Frequency Spectrum');

grid on;

% Mark carrier, USB, and LSB frequencies

hold on;

xline(LSB, '--r', 'LSB = 4500 Hz');

xline(fc, '--k', 'Carrier = 5000 Hz');

xline(USB, '--g', 'USB = 5500 Hz');

hold off;

Output:

Objective 3:

To calculate the carrier power, sideband power, and total transmitted power of the AM signal for

a given modulation index using MATLAB.

Program:

clc;

clear;

close all;

% Given parameters

Ac = 5;           % Carrier amplitude

mu = 0.2;         % Modulation index

R = 1;            % Load resistance in ohms

% Calculate carrier power

Pc = Ac^2/(2*R);

% Calculate sideband powers

P_USB = (mu^2*Pc)/4;

P_LSB = (mu^2*Pc)/4;

% Total sideband power

P_SB = P_USB + P_LSB;

% Total transmitted power

P_total = Pc + P_SB;

% Display results

fprintf('Modulation Index = %.2f\n', mu);

fprintf('Carrier Power = %.3f W\n', Pc);

fprintf('USB Power = %.3f W\n', P_USB);

fprintf('LSB Power = %.3f W\n', P_LSB);

fprintf('Total Sideband Power = %.3f W\n', P_SB);

fprintf('Total Transmitted Power = %.3f W\n', P_total);

% Plot power distribution

figure('Name','AM Power Calculation','NumberTitle','off');

power = [Pc P_USB P_LSB P_total];

labels = {'Carrier','USB','LSB','Total Power'};

bar(power);

set(gca,'XTick',1:4);

set(gca,'XTickLabel',labels);

ylabel('Power (W)');

title('AM Signal Power Distribution');

grid on;

output:

OBJECTIVE 4

iv) To analyze the effect of varying the modulation index on sideband power distribution and

overall transmission efficiency through MATLAB simulation.

PROGRAM:

clc;

clear;

close all;

% Parameters

Ac = 5;              % Carrier amplitude

R = 1;               % Load resistance in ohms

mu = 0:0.1:1;        % Modulation index from 0 to 1

% Carrier power

Pc = (Ac^2)/(2*R);

% Calculate sideband and total power

P_USB = (mu.^2 * Pc)/4;

P_LSB = (mu.^2 * Pc)/4;

P_SB = P_USB + P_LSB;

P_total = Pc + P_SB;

% Calculate transmission efficiency

efficiency = (P_SB ./ P_total)*100;

% Display results

fprintf(' AM Modulation Index Analysis\n');

fprintf('---------------------------------------------\n');

fprintf(' mu    USB(W)   LSB(W)   Total SB(W)   Total P(W)   Efficiency(%%)\n');

fprintf('---------------------------------------------\n');

for k = 1:length(mu)

fprintf('%.1f   %.3f    %.3f      %.3f         %.3f        %.2f\n', ...

mu(k), P_USB(k), P_LSB(k), P_SB(k), ...

P_total(k), efficiency(k));

end

% Plot 1: Sideband power distribution

figure('Name','Sideband Power Distribution', ...

'NumberTitle','off');

plot(mu, P_USB, '-o', 'LineWidth', 1.5);

hold on;

plot(mu, P_LSB, '-s', 'LineWidth', 1.5);

plot(mu, P_SB, '-^', 'LineWidth', 1.5);

xlabel('Modulation Index (\mu)');

ylabel('Power (W)');

title('Effect of Modulation Index on Sideband Power');

legend('USB Power','LSB Power','Total Sideband Power', ...

'Location','northwest');

grid on;

hold off;

% Plot 2: Total transmitted power

figure('Name','Total Transmitted Power', ...

'NumberTitle','off');

plot(mu, P_total, '-o', 'LineWidth', 1.5);

xlabel('Modulation Index (\mu)');

ylabel('Power (W)');

title('Modulation Index vs Total Transmitted Power');

grid on;

% Plot 3: Transmission efficiency

figure('Name','Transmission Efficiency', ...

'NumberTitle','off');

plot(mu, efficiency, '-o', 'LineWidth', 1.5);

xlabel('Modulation Index (\mu)');

ylabel('Efficiency (%)');

title('Modulation Index vs Transmission Efficiency');

grid on;

output: