EXP. NO:3

MATLAB-BASED MODELLING OF VSB MODULATOR AND DEMODULATOR FOR ANALOG TELEVISION COMMUNICATION APPLICATIONS

Objectives:

i) To generate and analyse the Vestigial Sideband (VSB) modulated signal by performing

DSB-SC modulation and applying a suitable filter using MATLAB.

PROGRAMMING CODE:

clc;

clear;

close all;

% Parameters

Am = 1;                 % Message amplitude

fm = 500;               % Message frequency (Hz)

Ac = 5;                 % Carrier amplitude

fc = 5000;              % Carrier frequency (Hz)

fs = 100000;            % Sampling frequency

T = 0.02;               % Signal duration

% Time vector

t = 0:1/fs:T-1/fs;

N = length(t);

% Message signal

m = Am*cos(2*pi*fm*t);

% Carrier signal

c = Ac*cos(2*pi*fc*t);

% DSB-SC modulation

dsb_sc = m .* c;

% Frequency axis

f = (-N/2:N/2-1)*(fs/N);

% Frequency spectrum of DSB-SC

DSB_FFT = abs(fftshift(fft(dsb_sc)))/N;

% Design VSB filter in frequency domain

H = zeros(1,N);

% Keep the positive-frequency sideband around the carrier.

% Retain the full USB and a portion of the LSB.

for k = 1:N

if abs(f(k)) >= fc && abs(f(k)) <= fc + fm

H(k) = 1;

elseif abs(f(k)) >= fc - fm && abs(f(k)) < fc

H(k) = 0.5;

elseif abs(f(k)) >= -(fc + fm) && abs(f(k)) <= -fc

H(k) = 1;

elseif abs(f(k)) > -fc && abs(f(k)) <= -(fc - fm)

H(k) = 0.5;

end

end

% Apply VSB filter

DSB_FFT_complex = fftshift(fft(dsb_sc));

VSB_FFT_complex = DSB_FFT_complex .* H;

vsb = real(ifft(ifftshift(VSB_FFT_complex)));

% VSB frequency spectrum

VSB_FFT = abs(fftshift(fft(vsb)))/N;

% Plot signals and spectra

figure('Name','VSB Modulation Analysis','NumberTitle','off');

subplot(3,2,1);

plot(t,m);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,2);

plot(t,dsb_sc);

title('DSB-SC Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,3);

plot(f,DSB_FFT);

xlim([fc-2*fm fc+2*fm]);

title('DSB-SC Frequency Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

grid on;

subplot(3,2,4);

plot(t,vsb);

title('VSB Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,5);

plot(f,VSB_FFT);

xlim([fc-2*fm fc+2*fm]);

title('VSB Frequency Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

grid on;

subplot(3,2,6);

plot(f,H);

xlim([fc-2*fm fc+2*fm]);

title('VSB Filter Response');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

grid on;

% Display information

fprintf('Message Frequency = %d Hz\n',fm);

fprintf('Carrier Frequency = %d Hz\n',fc);

fprintf('Lower Sideband Range = %d to %d Hz\n',fc-fm,fc);

fprintf('Upper Sideband Range = %d to %d Hz\n',fc,fc+fm);

fprintf('VSB signal generated successfully.\n');

OUTPUT:

OBJECTIVE 2

ii) To implement coherent demodulation and recover the original message signal from the VSB

signal, and compare the transmitted and recovered waveforms.

PROGRAMMING CODE:

clc;

clear;

close all;

% Parameters

Am = 1;                 % Message amplitude

fm = 500;               % Message frequency (Hz)

Ac = 5;                 % Carrier amplitude

fc = 5000;              % Carrier frequency (Hz)

fs = 100000;            % Sampling frequency

T = 0.02;               % Signal duration

% Time vector

t = 0:1/fs:T-1/fs;

N = length(t);

% 1. Generate message signal

m = Am*cos(2*pi*fm*t);

% 2. Generate carrier signal

c = Ac*cos(2*pi*fc*t);

% 3. DSB-SC modulation

dsb_sc = m .* c;

% 4. Design simplified VSB filter

f = (-N/2:N/2-1)*(fs/N);

H = zeros(1,N);

% Retain USB and half of LSB

for k = 1:N

if abs(f(k)) >= fc && abs(f(k)) <= fc+fm

H(k) = 1;

elseif abs(f(k)) >= fc-fm && abs(f(k)) < fc

H(k) = 0.5;

elseif abs(f(k)) >= -(fc+fm) && abs(f(k)) <= -fc

H(k) = 1;

elseif abs(f(k)) > -fc && abs(f(k)) <= -(fc-fm)

H(k) = 0.5;

end

end

% Apply VSB filter

DSB_FFT = fftshift(fft(dsb_sc));

VSB_FFT = DSB_FFT .* H;

vsb = real(ifft(ifftshift(VSB_FFT)));

% 5. Coherent demodulation

% Multiply VSB signal by synchronized carrier

demod = vsb .* cos(2*pi*fc*t);

% 6. Low-pass filter to recover message

[b,a] = butter(5, 2*fm/fs);

recovered = filtfilt(b,a,demod);

% 7. Normalize recovered signal

recovered = recovered - mean(recovered);

recovered = recovered / max(abs(recovered));

% 8. Plot results

figure('Name','VSB Coherent Demodulation', ...

'NumberTitle','off');

subplot(3,1,1);

plot(t,m,'b','LineWidth',1.2);

title('Original Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on

subplot(3,1,2);

plot(t,vsb,'r','LineWidth',1.2);

title('VSB Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on

subplot(3,1,3);

plot(t,m,'b','LineWidth',1.5);

hold on;

plot(t,recovered,'r--','LineWidth',1.2);

title('Original and Recovered Message Signals');

xlabel('Time (s)');

ylabel('Normalized Amplitude');

legend('Original Message','Recovered Message');

grid on;

hold off;

% 9. Display result

fprintf('VSB coherent demodulation completed.\n');

fprintf('Message frequency = %d Hz\n',fm);

fprintf('Carrier frequency = %d Hz\n',fc);

OUTPUT: